Compaq Super VGA chips.

3CEh index  Fh:

writing 5 to this register unlocks the extended registers.

3CEh index 45h: Bank register

bit 0-7  Bank number in 4k blocks.

ID Compaq super VGA:

rp.ax:=$BF03;

rp.bx:=0;

rp.cx:=0;

intr($10,rp):

if rp.ax=$BF03 then Compaq.

Video Modes:

2Eh G 640x480  256c  packed

59h G 800x600   16c  planar

Compaq BIOS extensions:

----------10BF00-----------------------------

INT 10 - VIDEO - Compaq Portable Extensions - SELECT EXTERNAL MONITOR

AX = BF00h

Note: all registers preserved and the internal monitor is blanked

the external monitor becomes the active monitor

SeeAlso: AX=BF01h

----------10BF01-----------------------------

INT 10 - VIDEO - Compaq Portable Extensions - SELECT INTERNAL MONITOR

AX = BF01h

Note: all registers preserved and the external monitor is blanked

the internal monitor becomes the active monitor

SeeAlso: AX=BF00h

----------10BF02-----------------------------

INT 10 - VIDEO - Compaq Portable Extensions - SET MASTER MODE OF CURRENT CTRLR

AX = BF02h

BH = master mode

04h CGA

05h EGA

07h MDA

SeeAlso: AX=BF03h

----------10BF03BX0000-----------------------

INT 10 - VIDEO - Compaq Portable/Systempro Extensions - GET ENVIRONMENT

AX = BF03h

BX = 0000h

Return: BH = active monitor

00h = external

01h = internal

BL = master mode

00h = switchable VDU not present

04h = CGA

05h = EGA

07h = MDA

08h = switchable LCD controller present

CH = 00h (reserved)

CL = switchable VDU mode supported

bit    0 = CGA supported

bits 1,2 = reserved (1)

bit    3 = MDA supported

bits 4-7 = reserved (1)

DH = internal monitor type

00h = none

01h = Dual-mode monitor

02h = 5153 RGB monitor

03h = Compaq Color monitor

04h = 640x400 flat panel

07h = LCD VGA

DL = external monitor type

00h = none

01h = dual-mode monitor

02h = 5153 RGB monitor

03h = Compaq Color monitor

04h = 640x400 flat panel

05h = VGC monochrome

06h = VGC color

SeeAlso: AH=1Ah,AX=BF00h,AX=BF01h,AX=BF02h

----------10BF04-----------------------------

INT 10 - VIDEO - Compaq Portable Extensions - SET MODE SWITCH DELAY

AX = BF04h

BH = new state of delay

00h enabled

01h disabled

SeeAlso: AX=BF05h

----------10BF05-----------------------------

INT 10 - VIDEO - Compaq Systempro Extensions - ENABLE/DISABLE DISPLAY

AX = BF05h

BH = new state of video

00h off

01h on

SeeAlso: AX=BF04h

----------10BF06-----------------------------

INT 10 - VIDEO - Compaq SLT/286 - READ GRAY SCALE TABLE

AX = BF06h

CL = address to be read from gray scale table

Return: AL = bit 3-0 - Value read from gray scale table

CL = address to be read from gray scale table

SeeAlso: AH=12h/BL=33h,AX=BF07h

----------10BF07-----------------------------

INT 10 - VIDEO - Compaq SLT/286 - WRITE GRAY SCALE TABLE

AX = BF07h

CH = value to write to gray scale table

CL = address to be written to gray scale table

SeeAlso: AX=BF06h

----------10BF08-----------------------------

INT 10 - VIDEO - Compaq SLT/286 - WRITE COLOR MIX REGISTERS

AX = BF08h

CH = bits 7-4 - Green weight

bits 3-0 - Blue weight

CL = bits 7-4 - unused

bits 3-0 - Red weight