This document describes how to do multiple outbound forwards.  The

normal mode is only one outbound forward which was the same mode as

before this feature was added.  If you are not using this feature then

the only difference will be that some of the prefixes on the messages

will be changed.

Groups

------

Forwarding is now done in groups.  There are a maximum of ten groups

numbered 0 to 9. The number of groups is set by the parameter

'MAXIMUM_FORWARD_GROUP' in PARMS.BB.  The default is 0 which means you

have only one group and the system behaves as it did without this

feature.

The number of groups is the MAXIMUM number of simultaneous forwards that

can be done.  Each group will need at least 20K of memory.

Ports

-----

Each port is assigned to a group using the 'FORWARD_GROUP' parameter in

PARMS.BB.  This parameter appears in the port section for each port.

All ports in a group will contend for forwarding.  If you have

Group 1  -- Port A and Port B

Group 2  -- Port C

then Group1 can forward either port A or port B but not both

simultaneously.  Port C can forward anytime without regards to ports A

and B.

Group 0 and Wakeup

------------------

The wakeup actions are run in group 0. If you want to be able to forward

out a port while a wakeup action is in progress, don't put that port in

group 0. In fact, I recommend no ports in group 0.

Command changes

---------------

The "X" commands now have a group number.

X[n] [path] [PORT p] [FORCE] [CONNECT]

Start a forward.

n -- optional group number.  The highest group number is

assumed if it is omitted.

path -- optional path pattern.  If specified, forward to

the paths specified.  Wildcards can be used.

PORT p -- option port pattern.  If specified, forward to

only the ports specified.  Wildcards can be used.

FORCE -- Force a connect to even if there are no messages

waiting.

CONNECT -- Allow a connect to even if this station is connected

on another channel/port.

Examples:

X

X3

X2 PORT B*

X3 WB6* CONNECT

X  WW6L FORCE CONNECT

X[n]STOP [ALL]

Stop forwarding

n -- optional group number.  The highest group number is

assumed if it is omitted.

ALL -- indicates that this will affect all groups.

Examples:

XSTOP

X1STOP

XSTOP ALL

Using WAKEUP to get more forwarding

-----------------------------------

Since the WAKEUP commands now run independent of the forward commands,

you can use WAKEUP to start forwarding.  Since you can define groups

with no ports, these are useful for extra forwarding.  A line like

931203/1437 ==/==/== +01:00 X5 WW6L

Will cause group 5 to forward to WW6L every hour independent of what the

other groups are doing.

There are some interesting possibilites with this feature.