UoSAT-OSCAR-14 PACSAT Communications Experiment Telemetry Format.

Telemetry is used to monitor the state of experiments and housekeeping

subsystems onboard the satellite.  In particular, it measures voltages,

currents, temperatures and the status of digital switches.

The PCE transmits telemetry in AX.25 UI frames at 9600 bps.  A "packet" of

telemetry is wholly contained in the information field of a single UI frame.

Each packet contains a timestamp indicating when the samples were taken, the

the data samples (including additional information detailed below), and a CRC.

The CRC is provided because the link between the KISS TNC used to receive the

I frames and the computer used to display or analyze the telemetry may become

unreliable at high data rates.  Thus the telemetry packet format is:

<timestamp> <data> [<data>...] <crc>

All of this is contained WITHIN the AX.25 UI frame information field.  The

information field will not exceed 256 bytes in length.

In addition to the fields contained in the telemetry packet, the receiving

process must maintain a <channel> variable.  The use of this variable becomes

clear below.

Interpretation of the fields is as follows:

BYTE ORDER

All multi-byte values are stored least significant byte first.  Where bits

are numbered, bit 0 is the least significant bit of a byte or multi-byte

value.

TIMESTAMP

The <timestamp> is a 4-byte, unsigned integer representing the number of

seconds since 0000 UTC January 1, 1970.  This is the UNIX standard time,

and is also  the format for times under the draft ANSI C standard.

DATA

Each <data> item is a 2-byte, unsigned integer, with internal encoding as

follows:

bits 0  - 11     A 12-bit <value> field

bits 12 - 15     A 4-bit <type> field

The <type> field tells the receiver how to interpret the <value> field, and

what other action to take.  The following <type> items are defined:

bit

15 14 13 12         Interpretation and Action

-- -- -- --         --------------------------

0  0  1  0         <value> is a channel number.

Initialize <channel> variable to <value>.

0  0  0  1         <value> is the telemetry sample for current <channel>.

Do not increment <channel> after processing <value>.

0  0  0  0         <value> is the telemetry sample for current <channel>.

Increment <channel> after processing <value>.

All other types are currently undefined and should be ignored.

The first <data> item in a telemetry packet will always be of type 0010, to

initialize the channel variable.

CRC

The cyclic redundancy check is provided as protection against errors on the

link between the receiving TNC and the computer which is processing the

telemetry.  It is the XMODEM CRC algorithm encoding all bytes, from the

first byte of the <timestamp> to the last <data> byte.

ADDRESS and PID

We are currently transmitting these frames with an AX.25 Protocol

Identification byte of 0xf0.  The source address is UOSAT3-0 and the

destination address is TLM-0.

EXPLAINATION and EXAMPLES

In the examples, assume that the <timestamp> is 2638d6ce hexidecimal. (From

here on, hexidecimal numbers are indicated by a leading 0x.)

You will see that this packet format allows us to efficiently encode the

most common type of telemetry "frame" - e.g. a frame in which we begin with

channel 0 and increment through the channel numbers.  In this case, the

packet starts out with:

0x2638d6ce    /* timestamp */

0x2000        /* <type>=0010, <value>=0 */

/* so set your channel variable to 0 */

and then proceeds with the data for each channel, with the <type> bits

indicating "increment" (0000). For example, if channel 0 is at a value of

0x496 and channel 1 is at 0x003, we have

0x2638d6ce    /* timestamp                      */

0x2000        /* set your channel variable to 0 */

0x0496        /* value of channel 0 and increment channel to 1 */

0x0003        /* value of channel 1 and increment channel to 2 */

Etc.

However, on UO-14 (as on UO-11) we have some "submultiplexed channels".

The primary example is channel number 15 which; every 12 times it is read

it returns voltages for the 10 cells of the battery and two 0 values for

synchronization.  Ideally we would have the PCE read the channel 12 times

for each telemetry packet and transmit all 12 values.  This is why the

"display but don't increment" <type> (0001) is defined.  So, now, in our

frame we have

0x2000        /* set your channel variable to 0                  */

0x0496       /* value of channel (0) and increment channel to 1 */

0x0001        /* value of channel (1) and increment channel to 2 */

...          /* 13 other <data> items with "increment" type.    */

...          /* leaving the channel variable at 15              */

0x10nn        /* first submultiplexed battery cell sample        */

0x10nn        /* second submultiplexed battery cell sample       */

...          /* and so on, until the last of the 12 samples,    */

0x00nn        /* which is of "increment type.                    */

We can also "jump over" any unwanted channels in the frame by inserting

another "set your channel variable" data entry.  This is done on UO-14 to

skip the blank channels between analog telemetry and digital status points.

So we have a flexible and efficient encoding system which does not require

the transmission of every channel number, yet permits submultiplexing and

channel number jumping.  In the case of UO-14, where the native telemetry

samples are 12-bits, we have "wasted" only 4 bits per channel on

supervisory information to arrive at this flexibility.

SAMPLE FRAME

A hex dump of the data in a telemetry packet from the UO-14 engineering

model is reproduced below.  It includes use of all features: 3 "set channel

variable" entries (one at the beginning of the frame and 2 to jump over

unwanted channels), 12 consecutive samples of channel 15 and 56 other

samples.  The CRC is also correct.

CE D6 38 26 00 20 00 00 16 02 37 00 07 00

CF 01 CF 01 CF 01 08 02 00 00 49 02 CB 00 2A 00

CF 01 CF 01 F4 01 33 12 32 12 30 12 2B 12 29 12

27 12 22 12 24 12 00 10 00 10 3A 12 34 02 00 00

6D 00 81 02 34 00 CF 01 CF 01 C8 01 81 01 54 01

2C 00 C7 01 04 03 CF 01 CF 01 CF 01 E6 01 B0 00

03 01 36 01 5D 01 6A 01 A1 01 CB 01 28 20 00 00

00 00 00 00 00 00 8F 01 FB 01 10 02 55 02 DD 00

40 20 80 00 00 08 02 00 80 00 12 08 83 00 10 04

08 08 00 08 AB A8

XMODEM  CRC

The XMODEM CRC receive checking is defined using the following C code:

unsigned short crc;              /* global crc register.              */

crc = 0;                         /* clear crc register                */

for (all received data bytes)

gen_crc(rx_byte);            /* crc the incoming data bytes       */

gen_crc(1st_crc_byte);           /* crc the incoming first crc byte   */

if(gen_crc(2nd_crc_byte))        /* crc the incoming second crc byte  */

�  bad_crc();               /* non-zero crc reg. is an error     */

else

good_crc();              /* zero crc is a good packet.        */

/* Function to take a byte and run it into the XMODEM crc generator.  */

/* Uses a global CRC register called "crc".                           */

gen_crc(data)

char data;

{

int y;

crc ^= data << 8;

for(y=0; y < 8; y++)

if( crc & 0x8000)

crc = crc << 1 ^ 0x1021;

else

crc <<= 1;

return crc;

}

Further information on generation of XMODEM CRC via more effecient byte-wise

methods can be found in BYTE Magazine,  September 1986, pp. 115-124.

CONCLUSION

Your comments are solicited - especially relating to any errors in this

document.  Address them to

Packet: G0K8KA @ GB3UP

Telemail: UOSAT

Other nets: ELE001@UK.AC.SURREY.SYSD

or, last resort

Jeff Ward, G0/K8KA

UoSAT Unit

University of Surrey

Guildford, Surrey GU2 5XH

United Kingdom