ROSERVER/PRMBS

Installation Guide

1. REQUIREMENTS

-  absolute  MINIMUM Requirements: IBM  compatible  computer

with 8088 CPU and 384K RAM, 10 megabyte HD (the system can be run

on  a  2x720K floppy drive system, please refer to  the  overview

manual  for detailed setup on 2 floppies) , 1  serial  communica-

tions port, DOS 3.2 or later, and a TNC1 or TNC2 compatible TNC.

-  suggested hardware: 8 MHz AT class machine 40 MByte  Hard

drive,  640 K RAM plus 2 MB EMS RAM, multiple  serial  communica-

tions ports, one TNC-2 for each serial port.

- what I would have if I owned Fort Knox: 386 or 486 machine

w/8 Meg RAM, QEMM-386 and Desqview. 4 serial ports and TNCs

2. PROGRAM FILES NEEDED

INSTALL.DOC    - this file

FIRSTRUN.BAT   - automated setup and install

RS180RUN.ZIP   - complete runtime executables and support files

RS177DOC.ZIP   - complete documentation

PKUNZIP.EXE    - file uncompaction utility

a. On your Hard drive create a directory called \PBBS beneath the

root  directory. Using CD (CHDIR) command make this your  current

working directory.

b.   Copy  all  of  the  above  files  to  this  directory.   The

FIRSTRUN.BAT  files  are  included inside  of  the  RS171RUN.ZIP.

Depending on how you obtained ROSERVER/PRMBS they may or may  not

already  be broken out. If they are not present copy  the  others

and then type;

PKUNZIP RS171RUN FIRSTRUN.BAT

This  will  extract those files that are needed.  You  will  then

execute the FIRSTRUN.BAT file;

FIRSTRUN 1 KA2BQE Brian Riley

| |      |     |

| |      |     +- sysop's last name

| |      +------- sysop's first name

| +-------------- sysop's call sign

+---------------- first message number in the mail file

Of  course you will replace my call with you own, my  names  with

yours.  I do not suggest that you use another number than  1  for

starting  unless  you are bringing up this system  subsequent  to

another working system and wish to follow its sequence numbers.

FIRSTRUN will extract all the necessary files, and will make

1

sub-directories  FILES,  MAIL,  SYS, and USR, as  well  as  place

certain files in the proper directories.

3. INITIAL Execution - CONSOLE only version

a.   Using  a  non-formatting  text  editor  to  open  the   file

CONFIG.CON.  Find the string  KA2BQE.#NWVT.VT.USA and replace  it

with  your call and H-address. If you are not sure  exactly  what

your local sub-region is make the address simply as;

yourcall.yourstate.yourcountry

Directly  beneath this entry in the config files are a series  of

H-addresses that are variants off of the H-address that is yours.

The  logic of the entries should be apparent. Change  only  those

things  needed for your location. Certain lines with  sub-regions

may  need  to be removed if you do not know or there is  no  sub-

region for your area.

b.   Execute  the file CONSOLE.BAT. After the program  stabilizes

and there is no more disk activity, hit the ESCape key the system

will  prompt you "login: " simply hit <ENTER> and to  the  "pass-

word: " prompt respond  PRMBS<ENTER>. You will now be at the  BBS

local console prompt. Type the following;

ADDRESS "your street address"<ENTER>    {optional}

QTH "your city, your state"<ENTER>      {mandatory}

ZIP yourzip<ENTER>                      {optional}

PHONE ###-###-####                      {optional}

Then type  X<ENTER> which will log you off and exit the BBS. This

step is necessary to get the location of the BBS into the sysop's

record for the sake of the BBS and WP activities.

c.   Re-Execute CONSOLE.BAT and re-login. When you reach the  BBS

prompt you will probably wish to change your password;

PASSWORD newpassword

d.   A  note  here. By typing a simple <ENTER>  at  the  "login:"

prompt  you are indicating that you  wish to login as the  'super

user'. You still must enter the correct password and the call  of

the sysop of the system is hardly a secret. had you entered  your

own  call  in at the "login:" prompt and then the  password,  you

would  enter  into the BBS as a plain vanilla user, able  to  see

only  bulletins and messages addressed to or from you and  unable

to do most of what the 'superuser' can do.

By way of seeing the difference in commands login as  super-

user  and at the BBS prompt, type CMDS (prior to  executing  CON-

SOLE.BAT  type  ctrl-P and all the screen output will go  to  the

printer) and see the list of available commands. Log out and exit

2

and  log back in with callsign and password to come in as a  peon

and then type CMDS and note the difference in the commands avail-

able to the plain user.

(*** NOTE *** when logged in as 'superuser' you can achieve  this

without  logging  out by typing CMDS -U which shows  normal  User

commands, or CMDS -S which shows remote Sysop commands)

Take some time and play with the commands, create  messages,

delete  messages etc. You wish to print out the Reference  manual

which  has  detailed  summaries of commands as a  guide.  I  call

particular  attention  to the E commands as this feature  is  not

available  in any other BBS but will make your life as  a  simple

PMS or a full-blown PBBS sysop very easy.

4. Bringing up an on the air version

a.   It  may be your intention to bring up a multi-connect  G8BPQ

Node  emulaton version, but you should learn to walk  before  you

run.  Included with the RUNtime package is a copy of MBBIOS  con-

figured for COM1. MBBIOS is a terminate and stay resident  commu-

nications  port  manager that provides  extended  INT14  support.

(almost all BBS programs are based around extended Communications

BIOS  programs like MBBIOS, FBIOS, et al.) If COM1 is not  avail-

able for your TNC you must use MBBCONFG to re-configure that copy

of MBBIOS for COM2 or whatever.

b.  Find the file CONFIG.RS open it in your editor and  make  the

same changes as to CONFIG.CON. The CONFIG.RS file has commands in

it for a standard TNC2. You need change only the Baudrate on  the

first  line of the PORT A section from 9600 to whatever you  have

set  up.  If you are using COM2 instead of COM1 then  change  the

PORT A to PORT B.

c.   Connect the TNC to the computer with the proper serial cable

that has TxD,RxD,DTR,RTS,CTS,DSR,DCD and GND wired up. Also check

to  see  if  your TNC has a jumper on it that  controls  DCD.  It

should  be  set so that if it supplies DCD it should  be  of  the

sense  that DCD is asserted when a station CONNECTs and  is  lost

when  DISConnected. If your TNC does not support this change  the

entry in the PORT configuration for DCD to NO.

d.  Count  to  10, execute RADIO.BAT and hold  your  breath!  The

system  should come up with a full windowed screen and  a  window

should  open  showing initialization commands being sent  to  the

TNC.  If the system has frozen without TNC responses,  check  the

cable and port assignments.

e.   To  check TNC connectivity, type TERMINAL A (or B or C  wha-

tever  port you configured), you should get a "cmd:"  prompt  and

then type a  C TEST<ENTER> and observe to see if the TNC tries to

transmit  by checking the PTT light. To exit from  TERMINAL  mode

hit the ESCape key.

f.   Connect up your radio and get someone to connect to you  and

3

check  to  see that the BBS recognizes the connect and  logs  the

user in.

5. When the test user above disconnects and the BBS goes back  on

line  simply type an X while the BBS is in monitor mode  awaiting

the next connect and it will exit. (this may seem awfully  'frag-

ile', but if you read the main overview manual you will find  out

that  you can activate a 'console lock' by typing ctrl-L  in  the

monitor/awaiting  connect  mode  so that the  keyboard  will  not

respond  to  any keystrokes except another ctrl-L to  unlock  the

keyboard)

6.  Now is the time to delve into the documentation and  see  the

workings  of  message  routing and  the  hierarchical  addressing

schemes, the EVENT processor, and the MESSAGES.RS files. You will

want to customize them for you system and locale.

7. When the time comes to actually run the program full time  you

will  want  to  run the BAT file  PBBS.BAT.  After  familiarizing

yourself with the docs and the details of running you may want to

make chnages in the BAT file to suit your individual tastes.

de KA2BQE

Brian B. Riley

P.O. Box 188, Harvey Rd.

Underhill Center, Vermont 05490

4